1. Introduction

Task 2 is performed in a factory scene simulated by Gazebo, where the robot-assistant is helping the workers deal with some tasks. With a closer look, the robot first explores the area, so it has a better understanding of its surroundings, then it finds the workers one by one where it iteracts with them via a QR code or directly through a TTS dialogue which are information about tasks the robot has to do. These tasks are either barrel, tile anomaly or ring detection where the robot constantly keeps track of the ojects it has detected so far. After completing every given task, it proceeds to move into a second room, where it solely navigates through the following of a blue line from the floor, where eventually it finds the factory’s CEO and hands him the report of every find it had on the previous room.

For this task, since we were a team with only 2 students, we skipped completely the tile anomaly detection. What we came up building was the robot localization, the face detection which contains the interaction with the workers, the ring and barrel detection, where for the rings we announce the color of the ring and for the barrels the shape of them; whether they are Horrizontal or Vertical and if the horizontal barrels are spilling or not, to maintain safety all across the factory. Finally, we also did the blue line follower where it constantly looks for the CEO to publish the report.

2. Methods

For the Barrel Detection:

Robot is constantly looking for specific coloured barrels with already specific given dimensions and other variables (I.e.min/max area of the barrels, depth_max for too-far rejections) for cv2 to look for. Because of the node complexity, all of the functions are private because we don’t want to call those functions outside of the BarrelDetection class sp there are 4 callback functions, 4 iterating functions working based on the callback ones and 4 plain.

More specifically, the callback functions rgb_cb and depth_cb connect with cv2 to pass info about the color and depth of the barrels, while barrel_report_cb is for display purposes and enable_cb is used to let the whole node sit idle when it is not needed, for example when we use the blue line follower.

For the 4 tick functions, detect_tick uses many sanity checks to determine what a barrel is -just color and height/width were not robust at all. display_tick displays the cameras that get updated live from the robot and it runs independently from the detect_tick. Lastly, approach_tick is when robot approaches a barrel, how it reacts based on the state of the barrel and stop is for the robot to terminated.

The 4 last functions are publish_detections where we save and store the barrels coordinates for the final report, find_barrel_core_bbox where we calculate the dimensions and color for the boundary box that is being shown live in gazeebo, check_spill where we make robust calculation checks for the horizontal barrels if they are leaking or not and publish_markers where we create markers in Rviz map.

For the Blue Line Follower:

The idea behind this node are the edges and vertices. Robot is navigating only through the top camera facing down so it only calculates how many lines it sees. The robot starts with watching 1 line (from now called edge) until it reaches a line split (from now called vertex), where it evaluates the position of the 2 new edges and turns into the left one continuing its journey. If it reaches to a dead end, it turns around and moves back to the latest explored vertex, where it turns to the right edge moving forward until it reaches a new vetrex where it executes the same procedure, until it detects the CEO to deliver him the report. In the code we have 8 option states:

1. CENTERING where the robot positions itself in the blue line so robot does not deflect when the line gets curved to either sides

2. FOLLOWING is the most active state where the robot is at the exploration stage, which is just constantly following the blue line until the CEO is detected

3. TURNING_AROUND where the robot has found a dead end and completes a 180° turn.

4. RETURNING is after robot reached a dead end, turned around and now it travels back to the last explored vertex.

5. DONE is the final state of the robot, where essentialy it tells the rest of the system it’s finished and the program terminates.

6. SPINNING_LEFT might look similar to number 3 but this one serves a different purpose, which is that the robot returned to the last explored vertex and needs to rotate until it’s facing the other unexplored edge, which then proceeds to follow.

7. VERTEX_PAUSE is when the robot faces a new vertex, it runs while robot is moving towards the highest point of the vertex then it calculates where to turn (auto turns to the left at first encounter)

8. APPROACHING_PERSON is when the robot, with the help of FaceDetection node finds the CEO and approaches him, while still following the blue line. By the time this state starts, FOLLOWING state terminates.

Since we know from the instructions that CEO is always going to be somewhere in the area, before we finish with the complete searching of the map, there is no need to have any failsafe states where robot returns back to starting position or even stops working on the stop. Robot constantly moves until CEO is detected.

For the Face Detection:

Very simple node, essentially we utilize the yolov8n.pt detector, a pre-trained object detection model that makes this whole process much easier. After detecting a face, robot marks it with a boundary box and draws it on cv2 and a marker gets added on the person’s position in Rviz, that came from the depth point cloud